# Deep Learning Digital Pre-Distortion for Power Amplifier Linearization

A Neural Network Approach to Nonlinear PA Compensation

Technical Implementation Report December 29, 2025

Abstract

This report presents a deep learning–based power amplifier (PA)  used for training digital pre-distortion (DPD) models to linearize power amplifiers in modern communication systems. The implementation employs a deep neural network to simulate the behaviour of a nonlinear physical PA exhibiting memory effects. Various DPD architectures, including feed forward neural networks, recurrent neural networks (RNN), gated recurrent units (GRU), and long short-term memory (LSTM) networks, are trained and compared to determine their suitability under identical PA conditions. Experimental results using 16-QAM signals demonstrate significant performance improvements, with NMSE reduction of 15–20 dB and EVM improvement of 20–25%. This study provides a systematic framework for comparing DPD models under controlled conditions and offers insights relevant to practical transmitter linearization in satellite and wireless communication systems.

Contents

- Introduction	3

  - Background	3

  - Motivation	3

- Technical Definitions	3

  - Digital Pre-Distortion (DPD)	3

  - Power Amplifier Nonlinearities	3

    - AM-AM Distortion	3

    - AM-PM Conversion	4

    - Memory Effects	4

  - Performance Metrics	4

    - Normalized Mean Square Error (NMSE)	4

    - Error Vector Magnitude (EVM)	4

    - Adjacent Channel Power Ratio (ACPR)	4

  - 16-QAM Modulation	5

  - Root Raised Cosine (RRC) Filter	5

- System Architecture	5

  - Overall DPD System	5

  - Indirect Learning Architecture	5

- Deep Neural Network Architecture	6

  - Input Features: Memory Polynomial	6

  - Network Architecture	6

  - Training Procedure	6

    - Loss Function	6

    - Optimization	7

    - Learning Rate Scheduling	7

- Implementation Details	7

  - Signal Generation	7

  - PA Model Implementation	7

- Experimental Results	8

  - Simulation Parameters	8

  - Performance Comparison	8

  - Key Observations	8

- Pathway to TWTA Implementation in Satellite Communications	8

  - TWTA Characteristics	8

    - Nonlinear Behavior	9

    - Operating Constraints	9

  - Step-by-Step Implementation Roadmap	9

    - Step 1: TWTA Characterization (1-2 months)	9

    - Step 2: Feedback Path Implementation (2-3 months)	10

    - Step 3: Enhanced Neural Network Design (2-3 months)	10

    - Step 4: Training Data Collection (1-2 months)	11

    - Step 5: Real-Time Processing Implementation (3-4 months)	11

    - Step 6: Adaptive Learning and Online Updates (2-3 months)	12

    - Step 7: Integration with Satellite Modem (2-3 months)	13

    - Step 8: Ground Station Validation (2-3 months)	13

# Introduction

## Background

Power amplifiers are essential components in wireless communication transmitters, responsible for amplifying signals to sufficient power levels for transmission. However, PAs exhibit nonlinear behavior, especially when operated near saturation for maximum efficiency. These nonlinearities cause:

- Spectral regrowth: Energy leakage into adjacent channels

- In-band distortion: Constellation warping and increased error rates

- Reduced system capacity: Higher bit error rates (BER)

Digital Pre-Distortion (DPD) is a baseband signal processing technique that compensates for PA nonlinearities by applying the inverse characteristic before amplification.

## Motivation

Traditional polynomial-based DPD methods have limitations in capturing complex PA behav-iors. Deep learning approaches offer:

- Superior modeling of complex nonlinearities

- Adaptive learning from data

- Generalization to various PA characteristics

- Real-time implementation feasibility

# Technical Definitions

## Digital Pre-Distortion (DPD)

Definition: A signal processing technique that applies an inverse nonlinear transformation to the input signal such that the cascade of the pre-distorter and PA results in an approximately linear system.

Mathematically, if the PA is characterized by y(n) = fP A(x(n)), the DPD function fDP D

satisfies:

fPA(fDPD(s(n))) ≈ G · s(n)	(1)

where s(n) is the desired signal and G is a constant gain.

## Power Amplifier Nonlinearities

### AM-AM Distortion

Definition: Amplitude-to-Amplitude distortion describes the nonlinear relationship between input and output signal amplitudes.

For input signal x(n) = A(n)ejϕ(n), the output amplitude is:

|y(n)| = g(A(n))	(2)

where g(·) is a nonlinear gain function exhibiting compression at high amplitudes:

αA

g(A) =	(3)

1 + βA2

### AM-PM Conversion

Definition: Amplitude-to-Phase conversion describes the phase distortion that depends on the signal amplitude.

The output phase includes amplitude-dependent distortion:

∠y(n) = ϕ(n) + ψ(A(n))	(4)

where ψ(A) is typically modeled as:

### Memory Effects

ψ(A) = −γA2	(5)

Definition: Memory effects occur when the PA output at time n depends not only on the current input but also on past inputs, caused by thermal, electrical, and trapping phenomena.

The PA with memory is modeled as:

Σ

M−1

y(n) =

hm(|x(n − m)|) · x(n − m)	(6)

m=0

## Performance Metrics

### Normalized Mean Square Error (NMSE)

Definition: Normalized power of the error signal relative to the reference signal power.

2

(7)

ΣN	|y(n) − x(n)|

Expressed in dB:

N

Σ

NMSE =   n=1

n=1

|x(n)|2

NMSEdB = 10 log10(NMSE)	(8)

Lower NMSE indicates better linearization (typical target: < −30 dB).

### Error Vector Magnitude (EVM)

Definition: RMS value of the error vector between ideal and measured constellation points.

u 1 Σ |Sn − Rn|2

N

n=1

|S

n

,,

N

EVMRMS =

|2	× 100%	(9)

where Sn are ideal symbols and Rn are received symbols. Standards requirements:

      - 16-QAM: EVM < 12.5%

      - 64-QAM: EVM < 8%

      - 256-QAM: EVM < 3.5%

### Adjacent Channel Power Ratio (ACPR)

Definition: Ratio of power in adjacent channels to power in the main channel, quantifying spectral regrowth.

ACPR = 10 log10

Padjacent	(10)

Pma

Typical requirement: ACPR < −45 dBc for 3GPP standards.

## 16-QAM Modulation

Definition: 16-Quadrature Amplitude Modulation encodes 4 bits per symbol using 16 constel-lation points arranged in a square grid.

Constellation points:

Si,q = (2i − 3) + j(2q − 3),	i, q ∈ {0, 1, 2, 3}	(11)

Normalized to unit average power:

norm

Si,q

Si,q	= √10	(12)

## Root Raised Cosine (RRC) Filter

Definition: A pulse-shaping filter used to limit signal bandwidth while minimizing intersymbol interference (ISI).

Impulse response:

√

1 +



π

2

π

1 − α + 4α

hRRC(t) =

α

2

t = 0

sin  π

+

π

cos  π

4α

(13)

4α

4α



1 −  2

sin[πt(1−α)/T ]+4αt cos[πt(1+α)/T ]

πt[1−(4αt/T )2]

where α is the roll-off factor and T is the symbol period.

# System Architecture

## Overall System

The complete system consists of three main blocks:

Input Signal → [DPD Block] → [PA] → Output Signal

↑	↓

[Training]

Figure 1: DPD System Architecture

otherwise

## NN PA modelling

The NN PA model learns from physical PAs Input and Output signals  .

fPA(NN)= fPA(real)

(14)

## NN DPD modelling

The NN DPD model learns from NN  PAs Output  and Input signals  .

fDPD(NN)=f -1PA(NN)					(15)

# Deep Neural Network Architecture for NN PA and NN DPD

## Network Architecture

The deep DPD network consists of:

- Input Layer: Feature normalization via Batch Normalization

ˆ	 f − µ

f = √σ2 + ϵ	(17)

- Hidden Layers: Dense layers with nonlinear activations

h(l+1) = LeakyReLU(BN(W(l)h(l) + b(l)))	(18)

Where LeakyReLU:

(

LeakyReLU(z) =	z	z > 0

0.2z	z ≤ 0

(19)

- Output Layer: Separate heads for I and Q components

I

Iˆ(n) = wT h(L) + bI	(20)

Q

Qˆ(n) = wT h(L) + bQ	(21)

Architecture summary:

  - Input dimension: 12 (memory depth = 5)

  - Hidden layers: [128, 64, 32] neurons

  - Dropout rate: 0.2

  - Activation: LeakyReLU(α = 0.2)

  - Output dimension: 2 (I, Q)

## Training Procedure

### Loss Function

Mean Squared Error between predicted and true signals:

1

L = N

N

Σ |	−	|

sˆ(n)	s(n) 2 = 1

N

n=1

N

Σ

[(Iˆn

n=1

- In

)2 + (Qˆn

- Qn

)2]	(22)

### Optimization

Algorithm: Adam optimizer with parameters:

      - Learning rate: η = 0.001

      - Momentum: β1 = 0.9

      - RMS decay: β2 = 0.999

      - Batch size: 128 Update rule:

mt = β1mt−1 + (1 − β1)∇θL	(23)

vt = β2vt−1 + (1 − β2)(∇θL)2	(24)

θt = θ

t−1

mt

t

— η √v + ϵ

(25)

### Learning Rate Scheduling

ReduceLROnPlateau scheduler:

ηnew = ηold × 0.5	if validation loss plateaus	(26)

# Implementation Details

## Signal Generation

16-QAM signal with RRC pulse shaping:

Algorithm 1 16-QAM Signal Generation

1: Generate random symbols: sk ∈ S16QAM

2: Upsample by factor Nsps: su[n] = s[n/Nsps] if n mod Nsps = 0

3: Apply RRC filter: x[n] = (su ∗ hRRC)[n] 4: Normalize peak: x[n] ← x[n]/ max |x[n]| 5: return x[n]

Parameters:

- Samples per symbol: 8

- Roll-off factor: 0.25

- Filter span: 10 symbols

## PA Model Implementation

The PA model includes memory effects and nonlinearities:

Table 1: Performance Metrics of Physical PA and NN-Based PA Model

| Metric | Physical PA | NN-PA |
| --- | --- | --- |
| NMSE (dB) | -10.58 | -11.65 |
| EVM (%) | 26.47 | 23.67 |
| ACPR (dB) | -43.01 | -37.37 |

Table 1: Simulation Configuration

| Parameter | Value |
| --- | --- |
| Number of symbols | 4000 |
| Samples per symbol | 8 |
| RRC roll-off | 0.25 |
| Memory depth | 5 |
| PA memory strength | 0.18 |
| PA saturation level | 1.5 |
| Training epochs | 100 |
| Batch size | 128 |
| Train/validation split | 70/30 |

# Experimental Results

## Simulation Parameters

## Input Features: Memory Polynomial

The network input consists of memory polynomial features to capture temporal dependencies:

x(n) = [x(n), x(n − 1), . . . , x(n − M )]	(15)

Each complex sample is decomposed into real and imaginary parts:

(n) = [ℜ{x(n)}, ℜ{x(n − 1)}, . . . , ℑ{x(n)}, ℑ{x(n − 1)}, . . .]	(16)

Feature dimension: Din = 2(M + 1) where M is memory depth.

## 6.3 Performance Metrics for DPD

### 6.3.1 Feed Forward NN

The original DPD implementation was used without modification, where I/Q samples are processed independently using fully connected layers. This model serves as the baseline for comparison.

| Metric | Without DPD | With Deep DPD | Improvement |
| --- | --- | --- | --- |
| NMSE (dB) | -10.58 | -24.64 | 14.06 dB |
| EVM (%) | 26.47 | 5.18 | 21.29 % |
| ACPR (dB) | -28.75 | -35.88 | 7.13 dB |

## 6.3.2 RNN

The feedforward layers were replaced with a PyTorch nn.RNN layer, and the input was reshaped into sequential batches to enable temporal processing of I/Q samples..

| Metric | Without DPD | With Deep DPD | Improvement |
| --- | --- | --- | --- |
| NMSE (dB) | -11.57 | -21.08 | 10.50 dB |
| EVM (%) | 23.81 | 8.47 | 18.00 % |
| ACPR (dB) | -43.01 | -31.79 | -11.22dB |

## 6.3.3 LSTM NN DPD

The GRU layer was replaced with an nn.LSTM layer, adding explicit cell states for longer memory modeling, with no changes to the loss function or optimization settings.

| Metric | Without DPD | With Deep DPD | Improvement |
| --- | --- | --- | --- |
| NMSE (dB) | -11.57 | -29.17 | 18.59 dB |
| EVM (%) | 23.81 | 2.89 | 23.58 % |
| ACPR (dB) | -43.01 | -37.67 | -5.34dB |

## 6.3.4  Gated Recurrent Unit (GRU NN)

The RNN layer was replaced with an nn.GRU layer, introducing gating mechanisms while keeping the same input sequence format, training procedure, and output mapping.

| Metric | Without DPD | With Deep DPD | Improvement |
| --- | --- | --- | --- |
| NMSE (dB) | -11.57 | -30.78 | 20.19 dB |
| EVM (%) | 23.81 | 2.39 | 24.08 % |
| ACPR (dB) | -43.01 | -38.14 | -4.87dB |

## 6.4 Observations

| Model | NMSE (dB) | EVM (%) | ACPR (dB) | Key Observation |
| --- | --- | --- | --- | --- |
| FFN | -24.64 | 5.18 | -35.88 | Best for low-memory PAs |
| RNN | -21.08 | 8.47 | -31.79 | Moderate memory handling |
| GRU | -30.78 | 2.39 | -38.14 | Best overall trade-off |
| LSTM | -29.17 | 2.89 | -37.67 | High performance, higher cost |

# 7.Conclusion

- Feed Forward DPD is effective for low-memory PAs but fails to adequately suppress memory-induced distortion.

- Recurrent architectures outperform feedforward models by explicitly modeling PA memory effects.

- GRU-based DPD achieves the best overall trade-off, delivering the highest NMSE and EVM improvements with stable training and moderate complexity.

- LSTM provides comparable performance to GRU but at higher computational cost, making it less attractive for practical implementations.

- These results highlight that DPD architecture selection is critical and should be guided by PA memory characteristics and system constraints rather than PA modeling alone.